Shm-KX Firmware Update Procedure

1.Confirm the IP address of SHM-KX

First, you must check what the current IP address of the SHM-KX

Press the "Left" or "Right" button to navigate to the settings menu

Press “Enter”, then press "Switch" to move to the Communication settings

Press “Enter” again to enter the Communication settings submenu

Press "Switch" repeatedly to move the focus to the second page and check the current network address of the SHM-KX.

2. Set the network address of the computer.

The computer used to connect to the SHM-KX must be on the same network segment as the SHM-KX, or be able to ping it via routing.

Right-click the “Win” icon , then click “Network Connections”, “Network & internet” interface appears.

Click "Properties" to enter the computer's network settings interface, as shown in the figure below.

Note: If your computer has multiple network adapters, there will be multiple setting tabs in this interface. Please select the network port that is connected to the device for configuration.

Click "Edit" to define the IP address of the computer's network port.

Fill in the corresponding IP address, as shown in the figure below.

Set the computer's network to be on the same subnet as the SHM-K.

In this example, the IP address of the SHM-KX is 192.168.11.137, and the subnet mask is 255.255.255.0. Therefore, the IP address of the computer can be set to 192.168.11.5, with the subnet mask also being 255.255.255.0.

Note: The IP address of the computer must not be the same as the IP address of the device.

3. Upgrade the firmware

Execute “ShmKX-UpdateTool“

Input ShmKX’s Ip Address

Click Connect

Switch to Tab “Tools”，Select the upgrade package file

Click Upload, Tools will upload the file, and you will see “upload firmware file finish”

Then click “Execute:

If the upgrade is successful, you will see the following message: "Update successfully."

Upgrade complete. Click Reboot. Shm-KX

In principle, updating the firmware will not cause the configuration to be reset (though some specific firmware versions may restore certain configurations to their default state), requiring the user to reconfigure them.